GSH-PC                         - 1 -                       Norms_e.doc

----------------------------------------------------------------------

Description of the SSTV modes/speeds supported by GSHPC

========================================================

The following assumes a SSTV video signal showing a colour bar image

with the following order of colours (from left to right);

white / yellow / cyan / green / magenta / red / blue / black

1./ Martin Modes

============

Martin-Modes were developed by Martin H Emmerson. G3OQD.

Martin-Modes are all free-running (also called "New Modes").

Mode       | Colour |  Resolution #1 |  Resolution #2 |   format

Nickname   | seq.   | Pixel x Line   | Pixel x Line   |   B x H

-----------+--------+----------------+----------------+------------

Martin-M1: | G-B-R  |   320 x 256    |   320 x 256    |    4:3

Martin-M2: | G-B-R  |   160 x 256    |   160 x 256    |    4:3

Martin-M3: | G-B-R  |   320 x 128    |   320 x 256    |    4:3

Martin-M4: | G-B-R  |   160 x 128    |   160 x 128    |    4:3

#1 Resolution of transmitted image

#2 Displayed resolution

Green           Blue             Red

|<------------->|<------------->|<----------->|

+-------+       +-+ +-+ +-+ +-+ +---+   +---+

|       |       | | | | | | | | |   |   |   |

|       |       | | | | | | | | |   |   |   |

|       |       | | | | | | | | |   |   |   |      |

--+ ++       +-------+ +-+ +-+ +-+ +-+   +---+   +---+ ++

| |                                                | |

+-+                                                +-+

5ms|(synch)      M1/M3 3x147 ms                     |

5ms|(synch)      M2/M4 3x 73 ms                     |

|<---------------------------------------------->|

2./ Scottie Modes

=============

Scottie Modes were developed by E.T.J Murphy GM3SBC.

The (modified) Scottie Modes are Free-Running modes.

Mode       | Colour |  Resolution #1 |  Resolution #2 |   format

Nickname   | seq.   | Pixel x Line   | Pixel x Line   |   B x H

-----------+--------+----------------+----------------+-------------

Scottie-S1 | G-B-R  |   320 x 256    |   320 x 256    |   4:3

Scottie-S2 | G-B-R  |   320 x 128    |   160 x 250    |   4:3

Scottie-S3 | G-B-R  |   320 x 128    |   320 x 128    |   4:3

Scottie-S4 | G-B-R  |   160 x 128    |   160 x 128    |   4:3

Scottie-DX | G-B-R  |   320 x 256    |   320 x 256    |   4:3

#1 Resolution of transmitted image

#2 Displayed resolution

GSH-PC                         - 2 -                       Norms_e.doc

----------------------------------------------------------------------

Green             Blue             Red

|<------------->|<------------->| |<------------->|

+-------+       +-+ +-+ +-+ +-+    +---+   +---+

|       |       | | | | | | | |    |   |   |   |

|       |       | | | | | | | |    |   |   |   |

|       |       | | | | | | | |    |   |   |   |

--+       +-------+ +-+ +-+ +-+ +-+ ++   +---+   +--+-

|                               | |               |

|                               +-+               |

|       S1/S3    3 x 140 ms  +  9ms (synch)       |

|       S4       3 x  90 ms  +  9ms (synch)       |

|       SDX      3 x 347 ms  +  9ms (synch)       |

|<----------------------------------------------->|

3./ Wraase SC-2 Colour Modes

========================

SC-1 and SC-2 modes were developed by Volker Wraase in Kiel.

The old SC-1 modes are not supported by GSHPC.

The improved SC-2 modes are Free-Running modes.

Mode       | Colour |  Resolution #1 |  Resolution #2 |   format

Nickname   | seq.   | Pixel x Line   | Pixel x Line   |   B x H

-----------+--------+----------------+----------------+------------

SC-2  30s  | R-G-B  |   256 x 128    |   320 x 128    |   4:3

SC-2  60s  | R-G-B  |   256 x 256    |   320 x 256    |   4:3

SC-2 120s  | R-G-B  |   512 x 256    |   320 x 256    |   4:3

SC-2 180s  | R-G-B  |   512 x 256    |   320 x 256    |   4:3

#1 Resolution of transmitted image

#2 Displayed resolution

Red                 Green                Blue

|<-------------->|------------------------->|<------------->|

+---+   +---+   +------------+             +-+ +-+ +-+ +-+

|   |   |   |   |            |             | | | | | | | |

|   |   |   |   |            |             | | | | | | | |

|   |   |   |   |            |             | | | | | | | |

-+ ++   +---+   +---+            +-------------+ +-+ +-+ +-+ ++ +

| |                |                          |              | |

+-+                |                          |              +-+

5ms|(synch)         |                          |              |

|      58 ms     |  SC_2 30s  117 ms        |     58 ms    |

|      58 ms     |  SC_2 60s  117 ms        |     58 ms    |

|     117 ms     |  SC_2 120s 235 ms        |    117 ms    |

|     235 ms     |  SC_2 180s 235 ms        |    235 ms    |

|<-------------->|<------------------------>|<------------>|

GSH-PC                         - 3 -                       Norms_e.doc

----------------------------------------------------------------------

3. ROBOT Color Modes

=================

Robot modes were developed by ROBOT RESEARCH, INC. San Diego CA9211

(e.g. Robot-1200c). Originally Robot-Modes were not free-running modes,

modified Robot-1200c and compatible converters using Martin EPROMs and

high-performance oscillators can receive and transmit these modes using

the free-run facility. GSH-PC can receive all Robot modes as normal or

free-running.

Mode       | Colour |  Resolution #1 |  Resolution #2 |   format

Nickname   | seq.   | Pixel x Line   | Pixel x Line   |   B x H

-----------+--------+----------------+----------------+------------

ROBOT 12s  | Y-U-V  |   160 x 120 *A |   160 x 120    |   4:3

ROBOT 24s  | Y-U-V  |   320 x 120 *B |   320 x 120    |   4:3

ROBOT 36s  | Y-U-V  |   320 x 240 *A |   320 x 240    |   4:3

ROBOT 72s  | Y-U-V  |   320 x 240 *B |   320 x 240    |   4:3

#1 Resolution of transmitted image

#2 Displayed resolution

*A Typ-A: Colour information for each 2nd. line.

*B Typ-B: Colour information for each line.

Typ-A: Colour information (R-Y) in each odd line.

____

|    ----____                       ################

|            ----____              +################+

|                    ----____      |################|

--+ ++                            ------+                + +--

| |                                                    | |

+-+                                 | |                +-+

10,5ms| |   12s-Mode    90ms        4,5ms | |     45ms       |

7ms | |   36s-Mode    60ms          3ms | |     30ms       |

-/-/<------------------------------->/-/<-------------->|

B/W image  (Y)     col-sync.   R-Y

Typ-A: Colour information (B-Y) in each even line.

____                             _

|    ----____                    | |################

|            ----____            | +################+

|                    ----____    |  ################|

--+ ++                            ----+                  + +--

| |                                                    | |

synch.+-+                                 | |                +-+

10,5ms| |   12s-Mode    90ms        4,5ms | |     45ms       |

7ms | |   36s-Mode    60ms          3ms | |     30ms       |

-/-/<------------------------------->/-/<-------------->|

B/W image  (Y)     col-sync.   B-Y

Typ-B. Colour information (R-Y) and (B-Y) in each line.

___                                    _

|   ---___                  ###########| |############

|         ---___           +###########+ +############+

|               ---___     |###########   ############|

--+ ++                     -----+                          + +--

| |                                                      | |

synch.+-+                         | |           | |            +-+

12 ms | |   24s-Mode    88 ms     | |   44 ms   | |    44ms    |

12 ms | |   72s-Mode   138 ms     | |   69 ms   | |    69ms    |

/-/<----------------------->/-/<--------->/-/<---------->|

6 ms   R-Y    6 ms   B-Y

B/W image  (Y)    1.col-sync.   2.col-sync.

GSH-PC                         - 4 -                       Norms_e.doc

----------------------------------------------------------------------

Black/white Modes

==================

Robot modes were developed by ROBOT RESEARCH, INC. San Diego CA9211

(e.g. Robot-1200c). Originally the Robot-Modes were not free-running

modes. Modified Robot-1200c and compatible converters using Martin

EPROMs and high-performance oscillators can receive and transmit these

modes using the free-run facility. GSH-PC can receive all Robot modes

in normal or free-running configuration.

Mode       | Colour |  Resolution #1 |  Resolution #2 |   format

Nickname   | seq.   | Pixel x Line   | Pixel x Line   |   B x H

-----------+--------+----------------+----------------+--------

SC-1   8s  | Bright.|   128 x 128    |   160 x 128    |  1:1

SC-1  16s  | Bright.|   256 x 128    |   320 x 128    |  1:1

SC-1  16sq | Bright.|   128 x 256    |   160 x 256 *  |  1:1

SC-1  32s  | Bright.|   256 x 256    |   320 x 256    |  1:1

Robot  8s  | Bright.|   160 x 120    |   160 x 120    |  4:3

Robot 12s  | Bright.|   320 x 120    |   320 x 240    |  4:3

Robot 24s  | Bright.|   320 x 240    |   320 x 240    |  4:3

Robot 36s  | Bright.|   320 x 240    |   320 x 240    |  4:3

#1 Resolution of transmitted image

#2 Displayed resolution

*  SC-1 16s "quasi" can be received in SC-1 8s mode.

white

____----+

____----        |

black    ____----                |

--+ +----                        ++ +--

| |                             | |

+-+                             +-+

|                               |

SC-1   8s    |5ms           60 ms            |

SC-1  16s    |5ms          120 ms            |

SC-1  16sq   |5ms           60 ms            |

SC-1  32s    |5ms          120 ms            |

ROBOT  8s:   |10ms          66 ms            |

ROBOT 12s:   |7ms          100 ms            |

ROBOT 24s    |12s          100 ms            |

ROBOT 36s:   |12s          150 ms            |

|<----------------------------->|

GSH-PC                         - 5 -                       Norms_e.doc

----------------------------------------------------------------------

The Vertical Interval Signal (VIS)

==================================

The VIS-code, introduced by ROBOT, is used by lot of SSTV-stations to

indicate the used mode/speed at the beginning of the transmission.

The VIS-code, decoded by the receiving station, allows automatic

selection of the correct mode/speed. The VIS-code is part of the

vertical sync pulse, and consists of 10 bits each 30 ms long. The first

and last bit (start and stop bits) are represented by 1200 Hz tones.

The VIS-code is coded in the remaining 8 bits, including one (even)

parity bit. There are 128 possible combinations.

1900 Hz         1900 Hz

------------+ +------------+ +-start-bit                     +-

grey     | |    grey    | |   example-code = 43 dec.      |

| |            | | b1 b2 b3 b4 b5 b6 b7 parity   |

| |            | V                               |

| | "0"=1300 Hz|  +--+  +--+  +--+  +--+  stop   |

+-+     1200 Hz+--+  |  |  |  |  |  |  |  +--+---+

10ms "1"=1100 Hz      +--+  +--+  +--+  +--+

|           | |            |                             |   |

| ca.300 ms | | ca.300 ms  |      10x30ms = 300ms        | t*|

|<----------/-/<---------->|<--------------------------->|<->|

*t is the horizontal sync pulse of the first line, the length is

dependent on the current SSTV speed. This length is very impor-

tant for the correct horizontal image position.

The meaning of the bits (with some exceptions);

+---+---+---+---+---+---+---+---+

| P | b7| b6| b5| b4| b3| b2| b1| <--LSB  least significant bit

+---+---+---+---+---+---+---+---+

|   |   |   |   |   |   |   |

|   |   |   |   |   | +---+---+

|   |   |   |   |   | | 0 | 0 | -> Color composite video

|   |   |   |   |   | | 0 | 1 | -> b/w   Red image

|   |   |   |   |   | | 1 | 0 | -> b/w   Green image

|   |   |   |   |   | | 1 | 1 | -> b/w   Blue image

|   |   |   |   |   | +---+---+

|   |   |   |   |   |     Horizontal resolution

|   |   |   |   | +---+   =====================

|   |   |   |   | | 0 | -> 128/160  Pixel

|   |   |   |   | | 1 | -> 256/320  Pixel

|   |   |   |   | +---+

|   |   |   |   |         Vertical resolution

|   |   |   | +---+       ===================

|   |   |   | | 0 | -----> 128/120  lines

|   |   |   | | 1 | -----> 256/240  lines

|   |   |   | +---+

|   |   |   |         Manufacturer / mode

| +---+---+---+       ===================

| | 0 | 0 | 0 | --->  ROBOT

| | 0 | 0 | 1 | --->  Wraase SC-1

| | 0 | 1 | 0 | --->  Martin  (G3OQD)

| | 0 | 1 | 1 | --->  Scottie (GM3SBC) & Wraase SC-2

| | 1 | 0 | 0 | --->  AVT-Mode & Scottie-DX

| | 1 | 0 | 1 | --->  r.f.u. (reserved for future use)

| | 1 | 1 | x | --->  r.f.u.

| +---+---+---+

+------------------> Parity-Bit (Even)

GSH-PC                         - 6 -                       Norms_e.doc

----------------------------------------------------------------------

The complete list of the available VIS-codes;

The LSB of the VIS-code (bit-1) is sent first, as usual by serial

data communication. Logical 1 is represented by 1100 Hz and

logical 0 by 1300 Hz. The start and stop bits are modulated at 1200 Hz.

+---------------+---------------+----------+

| Norm-Nickname |msb  bits   lsb| decimal  |

|               | 7 6 5 4 3 2 1 |  value   |

+---------------+---------------+----------+

| ROB 12s col.  | 0 0 0 0 0 0 0 |     0    |      1: red image

| ROB  8s b/w   | 0 0 0 0 0 x x |   1,2,3  | ---> 2: green image

| ROB 24s col.  | 0 0 0 0 1 0 0 |     4    |      3: blue image

| ROB 12s b/w   | 0 0 0 0 1 x x |   5,6,7  |

| ROB 36s col.  | 0 0 0 1 0 0 0 |     8    |

| ROB 24s b/w   | 0 0 0 1 0 x x |  9,10,11 |

| ROB 72s col.  | 0 0 0 1 1 0 0 |    12    |

| ROB 36s b/w   | 0 0 0 1 1 x x | 13,14,15 |

+---------------+---------------+----------+

| 24s col SC-1* | 0 0 1 0 0 0 0 |    16    |

|  8s b/w SC-1  | 0 0 1 0 0 x x | 17,18,19 |

| 48s col SC-1* | 0 0 1 0 1 0 0 |    20    |

| 16s b/w SC-1  | 0 0 1 0 1 x x | 21,22,23 |

| 48q col SC-1* | 0 0 1 1 0 0 0 |    24    |

| 16q b/w SC-1* | 0 0 1 1 0 x x | 25,26,27 |

| 96s col SC-1* | 0 0 1 1 1 0 0 |    28    |

| 32s b/w SC-1  | 0 0 1 1 1 x x | 29,30,31 |

+---------------+---------------+----------+

| Martin-M4     | 0 1 0 0 0 0 0 |    32    |

| Martin-M3     | 0 1 0 0 1 0 0 |    36    |

| Martin-M2     | 0 1 0 1 0 0 0 |    40    |

| Martin-M1     | 0 1 0 1 1 0 0 |    44    |

+---------------+---------------+----------+

| Scottie-4     | 0 1 1 0 0 0 0 |    48    |

| Scottie-3     | 0 1 1 0 1 0 0 |    52    |

| Scottie-2     | 0 1 1 1 0 0 0 |    56    |

| Scottie-1     | 0 1 1 1 1 0 0 |    60    |

| Scottie-DX    | 1 0 0 1 1 0 0 |    76 #  |

+---------------+---------------+----------+

| SC2 col 30s   | 0 1 1 0 0 1 1 |    51    |

| SC2 col 180s  | 0 1 1 0 1 1 1 |    55    |

| SC2 col 60s   | 0 1 1 1 0 1 1 |    59    |

| SC2 col 120s  | 0 1 1 1 1 1 1 |    63    |

+---------------+---------------+----------+

| AVT col 24s * | 1 0 0 0 0 0 0 |    64    |

| AVT col 90s * | 1 0 0 0 1 0 0 |    68    |

| AVT col 98s * | 1 0 0 1 0 0 0 |    72    |

| AVT col 188s* | 1 0 0 1 1 0 0 |    76 #  |

| AVT b/w 125s* | 1 0 1 0 0 0 0 |    80    |

+---------------+---------------+----------+

# Scottie-Dx and AVT-188s share the same VIS-code.

* Not supported by GSH-PC.

- When in B/W mode GSH-PC always sends the "green" code.

Martin mode was developed by Martin H. Emmerson G3OQD /England.

SC-1 and SC-2 converters were developed by Volker Wraase /Germany.

The Scottie mode was developed by E.T.J. Murphy GM3SBC /Scotland.

ROBOT is a trademark of ROBOT Research Inc. /USA.