GSH-PC                         - 1 -                          GSH-2.2

----------------------------------------------------------------------

GSH-PC SSTV - CONVERTER

=======================

Copyright (C) 1995-1997 G.Szabados-Hann

All Rights reserved.

About this program

==================

GSH-PC is a program for sending and receiving SSTV (Slow-Scan

Television) pictures. This program is not a multi-mode decoder for

FAX/RTTY/PACTOR/etc., it was designed specifically for SSTV.

However, the program only requires a very simple interface circuit and

offers very good picture quality.

Features:

=========

- 14 thumbnail pictures,

- Split window for RX/TX,

- Multitasking, e.g. image loading during receiving,

- Supports all commonly used modes (ROBOT, SCOTTIE, SC-2 and MARTIN),

- Load in GIF, PCX, TGA, JPG, CMP, BMPand TIFF image formats,

- Save in JPG (JFIF 4:2:2), CMP, BMP and TIFF image formats,

- Needs a very simple hardware interface, only two op-amps,

- Up to 16 M. colours by VESA-norm. compatibility,

- Supports the real-time (VD-720) frame-grabber,

- Grafik user Interface with Buttons and 3-D effects.

Hardware requirements

=====================

1) PC and graphics adapter

=========================

The program runs on MS-DOS PCs with at least 640 KB memory.

It needs an 1MB VGA graphics adapter supporting the 640x480 VESA-mode,

15, 16 or 24 bits per pixel (VESA modes 272, 273 or 274).

It is suggested using at least a 386DX processor machine with VLB-VGA

board. Colour resolution is, depending on the VESA-mode, 32K, 64K

or 16M colours per pixel. The best available resolution will be

selected automatically. Additionally at lest 1 MB extenden memory

is necessary (suggested 4MB).

2) Interface

============

To convert the audio signal between receiver and PC, a very simple

interface circuit is used - see the proposed circuits in appendix-1

at the end of this manual. This kind of interface (Hamcomm type)

circuit is used by several other RTTY and SSTV programs.

GSH-PC                         - 2 -                          GSH-2.2

----------------------------------------------------------------------

3) Video Frame-Grabber

======================

Using the FG-1 (VD-720) digitiser board, you are able to freeze an

image from any video source (FBAS input). The image is loaded directly

into the TX-Window and can be transmitted immediately. Unfortunetly

this version in not more deliverable. This is an optional facility,

you can also operate GSHPC without the frame-grabber.

Installation of GSPHC

=====================

Copy the compressed file "GSHPCZIP.EXE" into the directory you want,

and start the program; for example: c:\sstv\ghspc. This file is a

self-extracting program, simply start the executable file. After

"unzipping", the following files are available:

Files:  (directory: \)

========

GSHPC.EXE       SSTV converter-program,

SSTV.BAT        Batchfile to easy start of GSHPC,

GSHPC_D.DOC     Operating manual (german),

GSHPC_E.DOC     Operating manual (english),

NORMS_D.DOC     Description of the SSTV mode (german),

NORMS_E.DOC     Description of the SSTV mode (english).

BIGTXT.TXT      Example text for genarating textfiles (8x4 chars.),

NORTXT.TXT      Example text for genarating textfiles (16x8 chars),

VESAMOD.EXE     Lists the supported VESA modes of the VGA adapter.

GSH-PC                         - 3 -                          GSH-2.2

----------------------------------------------------------------------

Starting GSHPC

==============

Before first starting GSHPC on your PC, you should delete any

configuration file MODE_DEF.CFG. This may possibly be an old version

from another user and could contain directories that do not exist on

your PC.

Start the program by typing:

GSHPC            <ret> ; if no text in the header,

GSHPC John       <ret> ; if text ohne space in the header,

GSHPC "op: John" <ret> ; if text with space in the header,

Starting by:

GSHPC "op: John" #272 <ret> or,

GSHPC "op: John" #273 <ret>

forces to switch to the VESA-mode 272 respectively 273. It can be

useful if you have problems with the true-colour mode (e.g. colour-

change).

Registration

============

At first start you are asked to enter your callsigna and the PIN-

number. If you do not have the pin, you can use the program as

test version. The only limitation, that the callsign "NONLIS" will

be overlayed when sending. Please order the PIN before going on-the-

air (see ordering information at the end of this docu).

If you have problems:

---------------------

- Check the VGA-adapter, use the VESAMOD.EXE and VGATEST.EXE programs.

- Install UNIVESA.EXE if the VGA-adapter does not supports VESA.

- Remove/disable unused drivers e.g. EMM-386, CD-ROM, Soundblaster.

- Slow-down the PC-clock or the internal bus (CMOS-configuration).

Operation:

=========

The program is controlled by function-keys and pull-down selection

lists. The main functions can be selected by pressing the first

(underlined) character of the function-name. If opened, the neigh-

bouring functions can be reached by means of the TAB or Curs <-/->

keys. After selecting the desired item (Pg-up/Pg-down), the selected

function is executed by pressing the RETURN key. Use the ESC key

to exit the function without making any changes. The thumbnails can

be selected by the cursor keys.

GSH-PC                         - 4 -                          GSH-2.2

----------------------------------------------------------------------

+--------------------------------------------------------------------+

|  Mode  Martin M1      | DL4SAW SSTV Converter | Version:  GSH 2.2  |

+-------+-------+-------+-------+-------+-------+------+------+------+

|  Mode | Load  | Save  | Fill  | Paint | Word  | Tx ##| Hold | Rx ##|

+-------+-------+-------+-------+-------+-------+------+------+------+

|Icon-1|      |      |      |     |      |      | Scope ::.     :    |

+------+------+------+------+-----+------+--RX--+ .. . .   . :..:... |

|Icon-8|      |      |      |     |      |Stack |     .     .        |

+----------------------------------+---------------------------------+

|     TX window  PICTURE.TIF       |     RX window  PIC_0001.BMP     |

+----------------------------------+---------------------------------+

|                                  |                                 |

|                                  |                                 |

|                                  |                                 |

|                                  |                                 |

|                                  |                                 |

|                                  |                                 |

|                                  |                                 |

|                                  |                                 |

|                                  |                                 |

|                                  |                                 |

|                                  |                                 |

|                                  |                                 |

+------+------+------+------+------+------+------+------+------+-----+

| /osc/| conf.| F-run| VIS  | Fsave| Tx<Rx| Tx>Rx| Zoom+| Scope|Help |

| F1 ##| F2 ##| F3 ##| F4 ##| F5 ##| F6   | F7   | F8   | F9 ##| F10 |

+----------------------------------+---------------------------------+

MAIN FUNCTIONS

--------------

Mode:   Selection of SSTV-mode (default = Martin-1).

=====   The file "norms.doc" shows an overview of the used SSTV norms.

Load:   Loads images in different image formats.

=====

To change the current drive or directory select the correspon-

ding item and press the <return> key. Subdirectories are

marked as <d>, upper directories as "..".

To load an image into the TX-window, select the file and press

the <return> key. Pressing Ctrl-<ret> loads the image into the

RX-window.

The files are listed in alphabetic order. Following image-

formats can be loaded:

* PCX, created by Zsoft, RLE-compressed,

* TGA, Targa image format uncompressed and RLE-commpressed,

* BMP, Microsoft/Windows including RLE-compressed formats,

* TIF, Tagged Image File Format, also LZW compressed formats,

* GIF, Graphics Interchange Format,

* JPG, JFIF, JTIF, YUV 4:4:4, 4:2:2 and 4:1:1, compressed,

* CMP, created by LEAD-Technologies Inc., compressed,

Some rarely used subformats cannot be loaded, please don't

surprise about it.

GSH-PC                         - 5 -                          GSH-2.2

----------------------------------------------------------------------

Text files in special GSHPC format (.txt) can also be loaded.

The example files "BIGTXT.TXT" and "NORTXT.TXT" show how to

compose these files. You can use any text editor to create and

change them.

Save:   Save images from the RX-window to disk:

=====

The current drive and the save-directory can be selected very

similar to the Load function. The selections will be stored

in the configuration file "MODE_DEF.CFG" and will be restored

even when restarting the program.

To store the content of the RX-window using the autoincrement

number as filename, select the number and press the <return>

key. When this number is selected, you can enter a filename.

The image will be stored on the preselected drive/directory

using this name. The default save image format respectively

compression degree (see also F2) can be overwritten pressing

the "<-/->" cursor keys. The selected image format is valid

only for this one session.

Pressing Ctrl-d resets the autoincrement number.

Fill:   Test cards:  The following built-in test cards are available:

=====

* Grey step scale.

* Colour bars.

* Test card for colour resolution.

* Test card with grey step and colour bars.

* Test card for horizontal resolution.

* Background. The R,G,B components can be adjusted

independently. The values 0-9 are allowe, "+" and "-"

are representing incrementing respectively decrementing

color components (from the top to bottom).

Paint:  Opening this window, you can select a colour and draw in the

======  TX-window using the mouse.  A standard mouse-driver should be

installed beforehand on the default COM-interface.

GSH-PC                         - 6 -                          GSH-2.2

----------------------------------------------------------------------

Word:   Enter text into the TX-window (16 x 8 characters).

=====

1) Available functions in Edit-Mode:

---------------------------------

* Cursor, Page-up, Page-down, Home, <-,->  Delete, End

* TAB           Change to Colour adjustment Mode.

* Alt-C         Delete text, the background is transparent.

* Alt-I         Append callsign. If entered on invoking the

program, the text (up to the first space)

will be copied.

* Ctrl-Return   Copy the text into the TX-window.

* Alt-N         Change character colors, next color set

* Alt-P         Change character colors, previous color set

8 user definable color sets are available.

2) Functions in Color Adjustment Mode;

----------------------------------

* TAB            Change to Edit-Mode.

* Cursor-Keys:   Select the color component to be adjusted.

* Page-up/down:  Change the value of the selected color

component.

* Alt-N          Change character colors, next color set

* Alt-P          Change character colors, previous color set

* CTRL-Return:   Enter the text into the TX-window.

All 8 character color sets can be changed individually by

the user. The new values are stored in the configuration

file MODE_DEF.CFG and will be restored when starting again

the program.

To set the "overlay" mode, one of the background color

values have to be set to "_".

Various color-effects can be reached setting incrementing

"+" or decrementing "-" color componnets.

Alt-W  The same function as Word inserting shadowed big characters

=====  (8 x 4) for better readibility.

Operating modes

================

Tx:   Transmitting images in the pre-selected SSTV mode

===

* Press the Tx-key once; the image will be sent only once, TX-LED is

yellow.

* Press the Tx-key again: Endless transmission, TX-LED is red.

* Break transmission;  press ESC key.

Hld:  Standby (default state),  All functions are available.

====

GSH-PC                         - 7 -                          GSH-2.2

----------------------------------------------------------------------

Rx:   Receives images in the pre-selected SSTV mode.

===

* Press the Rx-key once; continuous reception, RX-LED is green.

* In contrast to the Sharewareversion, almost all functions are

available during image receiving. Exceptions: slant correcture

and all operations writing in the RX-window.

Horizontal and vertical synchronisation

========================================

-  F1 H-Res: Horizontal reset / Search mode.

---------------------------------------------------

After starting the receive function, the program is looking for

synchronisation impulses according to the pre-selected mode.

In the case of noisy signals, or receiving in incorrect mode/

speed, the F1-LED is yellow, signalling that reception cannot

be started. If you are in the correct mode, the image begins to

to be written automatically after detection of the

synchronisation impulse. Alternatively, you can force a search

for the correct mode by means of pressing the F1-key. During

the search procedure, the F1-led is set to red. After a certain

time, depending on the signal quality, it switches to the

correct mode and starts image reception. The F1-LED is then set

to green. Each time the image has been synchronised incorrect-

ly, you can re-synchronise the image by pressing the F1-key.

During resynchronisation, the F1-LED is set to yellow.

There are some limitations, because some SSTV-modes use

the same horizontal frequency. The program cannot distinguish

between:

* Martin-1  and Martin-3    (selects M-1)

* Martin-2  and Martin-4    (selects M-1)

* Scottie-1 and Scottie-3   (selects S-1)

* Scottie-2 and Scottie-4   (selects S-1)

* 32s S/W   and 16s B/W     (selects 32s)

* 30s SC-2  and 60s SC-2    (selects SC-2 60s)

* 12s S/W   and 24s B/W     (selects 24s B/W Robot)

-  F2 V-Res: Vertical restart

--------------------------

Pressing the F2-key, the receive cursor (small green arrow)

jumps to the first line and horizontal resynchronisation

is forced.

The F2-LED shows the reason for the last vertical reset;

* LED-off;  Idle-state / manual restart by pressing "F2".

* LED-green;

- An Erroneous VIS-Signal was received (parity error)

- This mode/speed is not supported.

- There is no VIS-code in the vertical synchronisation signal.

* LED-red;

New mode/speed has been selected, caused by the VIS signal.

GSH-PC                         - 8 -                          GSH-2.2

----------------------------------------------------------------------

* LED-yellow:

- A valid VIS-signal was received, the mode/speed is not

changed.

- The VIS mode selection was barred (F4) or GSHPC was

already in the correct mode/speed.

The F2-LED is switched off after the 10th line.

-  F3 F-Run: Free-Run  (See F-key description)

-  F4 VIS:   Change of mode/speed on VIS signal enabled/disabled.

-  F9 Osc.:  Display-mode of the Scope function:

- Analogue            F9-LED: yellow

- spectrum analyser   F9-LED: green  (default)

- off                 F9-LED: off

FUNCTION KEYS

-------------

F1: Slant correcture - available only in the HOLD-State!

================

The variation in frequency between PC oscillators does not allow

one to receive pictures initially without some slant. For this

reason, you will have to calibrate the timing in the following

manner.

1) Receive an image in Martin-M1 or Scottie-S1 mode.

2) Press F1, to start the calibration.

3) Set the red line parallel to a vertical line in the image.

4) Press Ctrl-Return to store the corrected value.

Use Ctrl-D to set the default values.  Pressing ESC exits

the calibration routine without storing the new value.

If calibrated while in Martin-M1 or Scottie-S1, GSHPC computes the

calibration factors for all modes/speeds. This means that all

other modes can be received without slant. In case some small

individual correction of another mode is necessary, this can be

done by starting the correction routine whilst in this mode. This

correction is then stored for this mode only, not affecting the

other modes.

F2: Configuration

=============

The required parameter field can be chosen by Curs-Up/Down.

Use "Curs <-/->" to select the correct parameters.

The following parameters can be adjusted.

1) Serial Interface:  Valid values: COM-1 to COM-8.

2) COM-Interrupt-line (IRQ):

When changing the serial interface, a default value will be

selected. You can overwrite this field. It is very important

to select a conflict-free interrupt line. Be sure that no

other interrrupt source use the selected interrupt line. The

interrupt lines 9-15 (AT-IRQs) could not be tested, because

no serial interface was available supporting this lines.

GSH-PC                         - 9 -                          GSH-2.2

----------------------------------------------------------------------

3) SSTV output signal via:

- Off

- PC-speaker

- TxD pin of the selected COM-interface.

- PC-speaker and TxD pin of the selected COM interface.

4) PTT-Control:

- Off.

- RTS Pin of the selected COM interface.

5) Grey step header at the top of the SSTV image;

- Off:  Header is not present.

- On :  Header is present (not in ROBOT mode).

6) Show directory for the Load and Save Functions.

- Off:  Do not show load/save directory.

- On:   Show load/save directory.

7) Fast-Save on/off.

- OFF/FAST_SAVE:  Fast-Save (F5) is disabled/enabled.

8) Default image format for manual- and fast-save.

- JPG(JFIF), CMP, TIFF and BMP. For JPG and CMP three different

quality factors can be selected:

- Low-Q:    filesize ca.  5-10 Kbyte depends on the image.

- Medium-Q: filesize ca. 10-25 Kbyte depends on the image.

- High-Q:   filesize ca. 25-50 Kbyte depends on the image.

By pressing Ctrl-d you can restore the default parameter values.

Use ESC to exit the menu without storing the selected parameters.

The window delivers following additional informations:

* Current VESA-Mode

* Free conventional (DOS) memory (below 1Mb)

* Free XMS memory (extended memory)

* CPU/VGA speed-ratio relativ to a 486-DX 50MHz PC with

VLB-SVGA by Genoa using Cirrus-Logic chipset.

F3: F-Run  Free-Run Mode on/off

===========================

The "new" SSTV modes like Martin mode, can only be received in

Free-Run mode. In this mode it is not possible to switch off

the F-Run function.

To enjoy the advantages of receiving images without synchronisa-

tion disturbance, new SSTV modes were developed. These new modes

combine the advantages of SSTV and FAX. The horizontal

synchronisation impulse allows the receiver to resynchronise the

image during image transfer. Normally, the picture is synchro-

nised only at the beginning of the transfer. The receiver starts

running, then is de-coupled from the synchronisation impulses. It

is absolutely necessary for the receiver and transmitter to have

precise clock signals in these modes.

Also, the "old" SSTV-modes (B/W and ROBOT) can be received Free-

Run; assuming that both pieces of equipment are running at

precisely the same frequency.

GSH-PC                        - 10 -                          GSH-2.2

----------------------------------------------------------------------

F4: Automatic change of mode/speed on VIS-Signal enable/disable

===========================================================

A lot of SSTV transmitters use the VIS-signal (Vertical Interval

Signal) to force the receiver into the correct mode/speed.

The VIS signal is 300 ms long and modulated by an 8-bit code

in addition to a start and stop bit. See further information in

"NORMS.DOC".

F5: FSave: Fast-Save Function.

=========================

The Fast-Save function allows storing of received images without

the need of opening the save window. The Fast-Save function have

to be enabled previously. See also F2-key description.

Pressing the F5-key stores the content of the RX-window on the

preselected drive/directory using the auto-increment number as

filename.

F6:       Copies Rx-window to Tx-Window.

Shift-F6: Copies the content of the RX-window in the TX-window with

reduced size.

Alt-F6:   Screen-Shot into the TX-window.

F7:       Copies Tx-window to Rx-Window

Shift-F7: Copies the content of the TX-window in the RX-window with

reduced size.

Alt-F7:   Screen-Shot into the RX-window.

F8: Zoom  Rx-Window

===============

This function allows magnification of the image in the RX-window.

Pressing any key restores the main window. The image in the

Tx-window will be deleted.

Shift-F8  Zoom Tx-Window

==============

Similar to F8, this magnifies the image in the Tx-window.

Pressing any key restores the main window. The image in the

Rx-window will be deleted.

Ctrl-F8: Show fast video input / freeze

==============================

This function is available only when the digitiser board VD-720

has been installed correctly (addr = 300 hex).

* The first pressing of Ctrl-F8 shows the fast video signal bl/wh.

This is not a real-time facility, the scan rate depends upon

the speed of your PC (approx. 3 pic/s. using a 486DX-40).

* The second pressing of Ctrl-F8 causes the last video sequence to

be stored in true colour mode. The max. colour resolution is

16M colours.

GSH-PC                        - 11 -                          GSH-2.2

----------------------------------------------------------------------

F9: Scope:  Shows the SSTV video signal in two different modes

==========================================================

Pressing F9 selects the display mode;

* LED is off; Function is not activated.

* LED is green; AF-spectrum.

* LED is yellow; oscilloscope.

Using the oscilloscope function, the receiver have to be adjusted

so, that the video signal remains between 1500 and 2300 Hz.

Approximately +/-100 Hz deviation will be tolerated.

Note that displaying the video signal is an additional load

for the PC. If using a slower PC and/or receiving in 128 line mode

displayed full size, the PC can reach its speed limit.

In this case, the displaying of the oszilloscope is disturbed.

F10: Contex sensitive Help-System

=============================

Thumbnails/icons

================

The icon you will operate, have to be selected first using

the cursor keys. Note that the last icon (right, bottom) has

a special "rx-stack" function. When pressing the return key,

a selection window appears offering a lot of functions:

* load, save icon from/to the tx or rx window,

* load, save icon from/to the tx or rx window with zoom,

* load, save icon from/to files,

* delete icon.

You can select the desired function pressing the <ret> key.

All functinos can be called directly from main window using

hotkeys (e.g. ctrl-return for loading of icon into the TX-win.).

RX-stack

========

Received images are copied automatically into the extended memory.

The available memory limits the number of images can be stored.

The number of currently stored images is displayed in the right

upper corner of the icon. The received images can be called back

using the +/- keys (if the Rx-stack icon was selected before).

The main advantage of the RX-stack, that the SSTV reception running

in the background is not influenced. Note that these images are not

stored on your HDISK!

GSH-PC                         - 12 -                         GSH-2.2

----------------------------------------------------------------------

Copyright and registration notes

================================

(c) Copyright 1995 G.Szabados-Hann DL4SAW. All rights reserved.

This version is not Shareware! To use this program you need your

callsign and a PIN-number. You can obtain the PIN-number and the disk

for $50, additionally $5 for shipping (DM 80.-) at the following

addresse:

------------------------------------

Mark Hann

Am Zuendhuetle 7a

76228 Karlsruhe / Germany

Tel./Fax.: 0721 / 47-53-19

e-mail: dl4saw@compuserve.com

------------------------------------

Please do not forget to give your callsign when ordering!

Payment via money order, postal money order, EURO-checque, VISA and

DINERS cards. When paying with credit card, please admit the card-Nr.

and the expiration date of the card. In case of paying with cheque,

it is important that the sum is entered in the currency of the paying

bank (keeping the charge fee as low as possible).

The program is also available from following dealers:

=====================================================

Phil Perkins, Pervisell LTD.

8 Temple End, High Wycombe

Bucks HP13 5DR / England

e-mail: ham@pervisell.com

Price: L 34.99, credit cards will be accepted.

---------------------------------------------

TBL-CLUB Rue du Bas  (F6AIU)

F-7020 LA ROCHE MOREY / Frankreich

e-mail: 101323,1053@compuserve.com

---------------------------------------------

JM, Applicaciones Electronicas

Jose Angel Veloso Fdez.

Apdo-130

48960 Galdacano (Vizcaya),  Spanien

Tel: (94)457-12-08

---------------------------------------------

TED Service Limited (JA2GZZ)

791 Mishuku Susono-city, 410-11 / Japan

e-mail: ted@shizuokanet.or.jp

---------------------------------------------

This software, although thoroughly tested, may contain some bugs.

The software is provided "as is", with no warranty expressed or

implied.

You use this program at your own risk, the author is not responsible

for any damage resulting by the use of this software.

Mni 73'dx DL4SAW

All brand and product names appearing in this manual may be trade

marks of their respective companies.

GSH-PC                 Appendix - 1/1                           GSH-2.2

------------------------------------------------------------------------

Improved quality comparator interface

=====================================

Jumper   PC

Tx-Level         10K     10K                   o----> Speaker

+---------+--####-+--####------------------o

0.1uF  |         |       |                        o-+

<----||--->R 10K    ===     ===                         |

Tx-Mic.    |         |4n7    |4n7                       |

-+-       -+-     -+-     47uf      1N4148    |     RS-232

|---||-----+---|<|---+ |     9.pin.

+    |         | |     SUB-D

100k               +-------+         | |

+--####-------+       |       | MC-     | |

|             |   47k R       | 1458/2  | +--o (3) TxD

Rx  1uF   4k7  |  +----\     |       |       |         |

>---||---####--+--|-     \   |  1K   |     +-+--\      +----o (4) DTR

Audio-            | OP-1  >--+--####-------|-     \

input         +---|+     /           |     | OP-2  >--------o (6) DSR

|   +----/         50K R<----|+     /

4k7 R    MC-1458/1  Offset |     +-+--/        +--o (7) RTS

|                      R 47k   |           |

-+-                     |       |           |  o (5) GND

+    |       |  1N4148   |  |

|---||--+-------+----|>|----+  |

Tx-PTT                          47uF                     |  |

<---------------\|     2k2                               |  |

npn |-----####--------|<|-------------------+  |

/|                1N4148                    |

-+-                                    |------+

Fig-1:  Circuit diagram of the suggested interface

OP-1 acts as an amplifier, OP-2 as a limiter. The supply voltage is

delivered by the lines DTR (+) and RTS (-). The program

sets these outputs accordingly.

The RTS pin can be used, if programmed to do so, for PTT-control.

You have two methods of transmitting SSTV images using this modem.

- output via PC speaker,

- output via TxD pin of the serial interface.

in both cases the jumper and the program parameter "Transmitting"

must be adjusted correctly (see this manual, F-2 key).

You can also use a singel OP interface like HAMCOMM. It is only impor-

tant to keep the square-wave signal as symmetrical as possible.

Unfortunately, most op-amps, like the 741 have a big offset error. For

this reason, an additional offset adjustment is necessary. Minimise the

offset error when receiving a strong single carrier signal (e.g.

calibrator signal of your receiver). Enter the scope function (F9) and

monitor the video signal. Adjust the 50K offset trimmer for minimum

noise ripple.

GSH-PC                 Appendix - 1/2                           GSH-2.2

------------------------------------------------------------------------

Another possibility is shown in fig-2. Using the CMOS op-amps CA-3240

no offset adjustment is necessary. Two diodes are added to protect

the input circuit against high voltage.

Jumper   PC

Tx-Level         10K     10K                   o----> Speaker

+---------+--####-+--####------------------o

0.1uF  |         |       |                        o-+

<----||--->R 10K    ===     ===                         |

Tx-Mic.    |         |4n7    |4n7                       |

-+-       -+-     -+-       47uf    1N4148    |     RS-232

2x1N4148                         |---||---+---|<|---+ |     9.pin.

+-|<|-|    22K      1M                 +  |         | |     SUB-D

|        |-####-+--####-------+           |         | |

+-|>|-|         |             |           | CA-     | |

|       22K     |  CA-3240/1  |           | 3240/2  | +--o (3) TxD

|               |  +----\     |           |         |

|               +--|-     \   |   1K    +-+--\      +----o (4) DTR

|  1uF (bipolar)   | OP-1  >--+--####---|-     \

+------||-------+--|+     /             | OP-2  >--------o (6) DSR

|               |  +----/     |--####---|+     /

R 1K            R 22K             1K    +-+--/        +--o (7) RTS

Rx |               |                         |           |

>--+              -+-                        |           |  o (5) GND

+     |  1N4148   |  |

|---||---+----|>|----+  |

Tx-PTT                                 47uF              |  |

<---------------\|     2k2                               |  |

npn |-----####--------|<|-------------------+  |

/|                1N4148                    |

-+-                                    |------+

Fig-2:  Circuit diagram of the modified interface

Following HAMCOMM compatible modems can be used:

-----------------------------------------------

Comparator-Modem, Pervisell LTD.

8 Temple End, High Wycombe

Bucks HP13 5DR / England

http://www.pervisell.com

--------------------------------------

BP-2 and BP-2M Tigertronics / U.S.A

http://www.tigertronics.com

--------------------------------------

SUPERMODEM fr PR/CW/RTTY/FAX/SSTV

KDK SATCOM Daten- und Kommunikationssysteme

Blankensteiner Str. 70

D-45527 Hattingen / Germany

--------------------------------------

DC6SN Klaus-J Sauvant,

Im Groetchen 38,

D-52146 Wrselen-Bardenberg / Germany

--------------------------------------